Road Safety Performance Evaluation and Policy Making by Data Envelopment Analysis: a Case Study of Provincial Data in Iran

Hamid Reza Behnood*, Techno-Economic Road Safety Research Center, Department of Civil Engineering, Faculty of Engineering, Ferdowsi University of Mashhad, Mashhad, Iran, email: hr.behnood@gmail.com

Esmaeil Ayati, Techno-Economic Road Safety Research Center, Department of Civil Engineering, Faculty of Engineering, Ferdowsi University of Mashhad, Mashhad, Iran, email: e_ayati@yahoo.com

Elke Hermans, Hasselt University, Transportation Research Institute (IMOB), Wetenschapspark 5 bus 6, 3590 Diepenbeek, Belgium, email: elke.hermans@uhasselt.be

Mohammadali Pirayesh Neghab, Department of Industrial Engineering, Faculty of Engineering, Ferdowsi University of Mashhad, Mashhad, Iran, email: pirayesh@um.ac.ir

* Corresponding author. Tel.: +98 915 3143674

Abstract

Considering the need to invest on road safety programs, an efficiency analysis on the implemented countermeasures is required. The system presented in this study is designed to evaluate the efficiency related to the measures annually implemented throughout 30 provinces of Iran in two years 2008 and 2009. The model calculates a relative inefficiency index for each province and in each year, using Data Envelopment Analysis (DEA) method. Each province in each year is defined as a decision making unit (DMU) in DEA analysis. The inefficiency index is defined as the proportion of weighted sum of road fatality risk indices to the weighted sum of road safety performance indicators. The inefficiency rate for each DMU must be minimized by either decreased fatality indices and/or increased safety measures to an optimal extent. Using a dual model of the main DEA model for each DMU, a target setting task can be conducted by identifying the benchmarks as the leading entities. Moreover, some discussions are provided for the results concerning the efficient units and two-year comparison analyses.

Keywords: Road Safety, Data Envelopment Analysis, Target Setting, Risk Indices, Efficiency.

1. Introduction

Great deals of studies and governmental plans have been made in various countries as a means to optimally implement road safety measures. Road safety management practitioners have always attempted to portray an explicit perspective in both quantitative and qualitative road safety evaluation criteria by setting distinct strategies.

In this study, a set of information about previously implemented road safety measures is applied to evaluate their efficiency effects in decreasing road fatalities. The system is designed to evaluate the performance related to measures annually implemented throughout 30 provinces of Iran in two years 2008 and 2009, following which a planning and decision making process could be run using needs found as the output of the analysis. The model calculates a relative inefficiency index for each province and in each year using Data Envelopment Analysis (DEA) method. The inefficiency index is defined as the proportion of weighted sum of road fatality risk indices to the weighted sum of road safety performance indicators. The inefficiency index as the final output of DEA analysis is used as a criterion in evaluating the existing performances as well as a tool for prospective planning efforts in the sense that a higher index represents an unsafe province in a given year.

The information used in this study are those forming the input and output data in DEA analysis. The information include a set of road safety performance indicators most of which are advised to be collected by Iranian Road Safety Commission [1]. According to some criteria (mentioned in section 2), this study detects for a subset of road safety measures defined in IRSC report which can suit the analysis framework of the study. The subset of measures considered for each region in this study includes: (1) police operation, (2) treated black spots, (3) freeways and highways, (4) speed cameras, (5) emergency medical services, and (6) road lighting projects. Having accounted the fatality rates as the road safety outcomes, a performance analysis is done to evaluate how inefficiently road safety measures act in the regions. Indeed, the entities which have the lowest inefficiency rates will represent the successful provinces in the year under study. The inefficiency rate for each decision making unit (DMU) must be minimized by either decreased fatality indices or increased safety measures to an optimal extent. Finally, for each DMU, a prioritization task can be conducted by identifying the benchmarks as the leading entities.

An early study in road safety usage of DEA was presented by Cook et al. [2] to prioritize highway accident sites. Afterwards, Odeck [3] used DEA to investigate target achievements of the operational units of the Norwegian Public Roads Administration (NPRA) charged with traffic safety services. In previous studies about combining road safety information in a performance index, the DEA approach was found obviously valuable in the road safety context that results in a useful optimization problem [4]. According to the so advantageous application of DEA, several road safety studies have utilized the approach as a performance evaluation method. Run-de et al. [5] have carried out a DEA model to evaluate the traffic safety for five urban counties within a city in China. In 2009, a promising methodology to make road safety policies by DEA results was introduced through which a model was described as a means to set targets and prioritize the road safety needs on the basis of a benchmarking approach [6]. The prioritization technique in this study is the approach to be applied as the data envelopment analysis road safety (DEA-RS) model in the current study. In 2009, a simple DEA framework for road safety priority setting was applied for different geographical areas in Bangladesh [7]. Shen et al. [8] presented a more comprehensive form of the analysis as a generalized multiple layer data envelopment analysis model for hierarchical structure assessment. He has also adopted the DEA extensions in road safety risk evaluation and target setting by evaluating the fatality rates in term of exposure measures [9].

In the next section, a description of the road safety performance indicators is presented to figure out how input safety measures and output fatality risk indices are selected for the analysis. Section 3 provides some methodological issues to be described about data envelopment analysis. The DEA application to build up DEA-RS model is presented in section 4 with additional discussion on benchmarking and prioritization tasks, as well as further results made by the analysis. Finally, conclusions are summarized in section 5.

2. Input and Output Indicators

Three main functions of indicators as defined by Adriaanse [10] are: simplification, quantification and communication. By using indicators we try to capture complex phenomena in relatively simple terms. Indicators generally use simplification to make complex phenomena quantifiable in such a manner that communication is either enabled or promoted. Furthermore, these indicators can be used to compare countries (or the provinces as used in the current study), to rank them and to benchmark them [11]. In general, road safety performance indicator (RSPI) is defined as any measurement that is causally related to accidents or injuries, used in addition to a count of accidents or injuries in order to indicate safety performance or understand the process that leads to accidents [12]. The RSPIs can depict a clear image of investments, planning, traffic violation recordings, public training efforts, etc. throughout policy makers. Safety performance indicators refer to operators’ activities within a transport system, but need not be limited to this.

The RSPIs defined as inputs of this study can be categorized as implementation and policy performance indicators. The outputs are also defined as fatality risk indices that should be calculated on the basis of officially stated road fatality frequencies. However, some criteria apply to the selection of an appropriate set of indicators based on the list of indicators introduced by IRSC [1] in Iran:

- For each of the indices, at least one year of valid data for all provinces should be available. Furthermore, data from official reports that are annually issued by the governmental agencies are important requirements.

- The indices including the valid data for all provinces should be chosen for at least one year. The indices partially collected in some provinces or the information adopted from the lectures or meetings are not suitable for the analysis. The data should include the information cited in the official reports that are annually issued by the governmental agencies.

- The data should be manageable and implementable to be applied in planning and decision making tasks. These indicators are usually attributed to the technical and infrastructural highway transportation indices but not the behavioral and human factors. For example, the indices like the rate of seat belts fastened, alcohol or drug impaired drivers, extra load violations by heavy vehicles, or the speed violations are controlled by the mediator plans such as the traffic public training or enforcement tasks thereby they cannot be directly implemented by the executive authorities. In contrary, the measures like the removed black spots and the number of police stations per unit length of a region are the tasks that could easily be handled by executive agencies.

- Choosing a great deal of indices as the input data can make the analysis procedure complicated and disorder the required balance between the inputs and outputs of the DEA analysis as well. As an empirical criterion in DEA analysis, the following equation must be observed between the number of inputs and outputs [13]:

Number of DMUs ≥ 3 × (Number of Inputs + Number of Outputs)

Otherwise, much DMUs will be located on the efficient boundary and their efficiency rate will equal to 1.0 thereby the model resolution power will be reduced.

- The input data must be independent of each other. In other words, none of the indices used as the analysis input data should be a function of another one so that no co-linearity must exist in the data set.

Many studies have shown that the simultaneous implementation of a set of road safety measures can lead a system into sustainable form. An increase in an RSPI might not lead to any improvement in terms of final outcome regarding fatalities or injuries. Therefore, both SPIs and final outcome, collated according to SPIs, should be collected and analyzed. The combination of a set of SPIs might lead to very few fatalities [14].

According to the illustrations about the road safety performance indicators discussed in this section, the six indices attributed to six measures are introduced as the main input data to be used in the analysis:

- Police Operation (PO): the average number of highway police stations in each 100 kilometers,

- Treated Black Spots (BS): the average number of black spots treated in each 100 kilometers,

- Highways and Freeways (H_F): The combined index is the percentage of weighted sum of highway and freeway lengths to the total length of the roads. To this end, each road type is weighted by the road safety equivalent factor mentioned in Highway Pavement Rehabilitation Manual of Iran [15].

- Speed Control Cameras (SCC): the average number of fixed speed cameras in each 100 kilometers,

- Emergency Medical Services (EMS): the average number of roadside stations serving the emergency medical services in each 100 kilometers, and

- Road Lighting Projects (Li): the average length of the road equipped by lighting poles in each 100 kilometers.

This study considers the abovementioned indices as the input values in DEA analysis while the risk indices reflecting the fatality rates compose the output values in the study. Table 1 shows all input and output data used as the applied indices in the analysis. Table 2 indicates a low co-linearity between input data using R2 rates. Except few cases, very low co-linearity is observed between pairs of indicators.

The risk indices as the output of the DEA analysis are fatality rates involving two aspects of the road fatality risk: the risk per unit value of the mobility demand (FR1; fatality number per one million vehicle-kilometers traveled) and the risk per unit value of the road (FR2; fatality number per 100 kilometers of the roads). These two types of risk refer to all types of roads in a region as well as the mobility demand which includes the total vehicle-kilometers traveled. All data regarding the road safety performance indicators (i.e. the indices referring the countermeasures) are adopted from the annual reports published by the Road Maintenance and Transportation Organization [16] in Iran. Besides, the data representing the fatality rates are adopted from Iranian Road Safety Commission [1] in Ministry of Road and Transportation.

3. Data Envelopment Analysis

In recent years, the Data Envelopment Analysis (DEA) has been known as a useful method to evaluate the performance of decision making units. Review studies on three decades of practices in DEA application have created a mature perspective on methodology developments in a variety of industrial and managerial activities ([17] and [18]). The first concept of the DEA model was presented by Charnes, Cooper and Rohdes [19] so known as the CCR model. The CCR model applies the proportion of weighted sum of outputs (y) to the weighted sum of inputs (x) as a scale for the measure of efficiency for n decision making units. If each decision making unit (DMU) includes m inputs and s outputs of production, the linear programming form of the DEA model to estimate the corresponding efficiency is as following:

| (1) | Max: θo=u1y1o+u2y2o+…+usyso |
| --- | --- |
|  | Subject to: |
| (2) | v1x1o+v2x2o+…+vmxmo=1 |
| (3) | u1y1j+u2y2j+…+usysj≤v1x1j+v2x2j+…+vmxmj (j=1, 2, …, n) |
|  | v1,v2,...,vm≥0 |
|  | u1,u2,…,us≥0 |

By solving this program, the optimum value θo for the objective function and the optimum values vi and ur for the function coefficients can be estimated. The value vi is the optimum weight for the input value i whose magnitude states the importance of that element. As well, the value ur states a similar weight for the outputr. The practical form of the main road safety DEA model used in this study is shown by equations 5 to 7 and described in section 4.

4. Concepts and Models

4.1. Data Envelopment Analysis Road Safety Model

Road fatalities are undesired outcome of mobility which should be minimized by road safety planning efforts. Policy makers wish to reduce fatality risk by implementing a variety of road safety measures while still need to be conscious about the benefits motivated by each type of remedial actions. As an alternative for the term ‘efficiency’, an ‘inefficiency’ analysis can best suit the essence of the proportion which contains the fatality rates in nominator and road safety supplies in denominator. On the basis of this concept, mere increase in road safety production factors is not always a reason for increase in inefficiency value. To this end, a frontier can be defined for fatality reduction below which a set of possible combinations, so called production possibility set of road safety measures can be established. Using the applicatory DEA approach to evaluate the road safety performance in all provinces, it will be possible to calculate an inefficiency index that reflects the rate of changes in fatality outputs to changes in safety inputs. For doing so, the road safety inefficiency in each province and each year is defined in term of input and output values as follow:

- Input values include six road safety performance indicators defined at the end of section 2.

- Output values include the risk of fatality captured in two types involving the risk per mobility demand and the risk per unit road length.

- The Inefficiency Index in each DMU to be defined as below:

II=r=12WFIri=16WPIi										  (4)

where:

II= Inefficiency Index that must be minimized

WFIr = weighted fatality index for risk index type r

WPIi = weighted performance indicator for ith input

By such an equation, the decrease in inefficiency interpreted as increase in road safety can be achieved in two ways as below:

- By decreasing the fatality index; it might not be meaningful unless for comparing two different units: the unit with a lower fatality index and the same inputs (road safety measures) is less inefficient (or more efficient). Consider that fatality reduction is just the result that policy makers tend to.

- By increasing the road safety measures; road safety policy makers tend to improve safety by implementing several measures in agenda.

Both these two approaches will decrease the inefficiency while still help reduce fatality outcomes. But an extent (i.e. frontier) exists up to which the inefficiency reduction trend stops. The following DEA model can best help us achieve such an extent:

Min θ=u1y1+u2y2=r=12uryr									  (5)

Subject to:

i=16vixi=1											  (6)

r=12ury rj−i=16vixij≥∘ (j=1,2,...,60)							(7)

vi, ur≥∘

The variable yr in (5) represents the related fatality rates defined as two risk indices. Equation (5) is the objective function to minimize the value of inefficiency rateθ. The model implies the inefficiency to lay at a minimum rate of one, thereby the weighted outputs will always be greater than or equal to the weighted inputs as displayed in (10).

4.2. Benchmarking

Having found the inefficiency rates, less prosperous provinces in a given year can systematically be compared with the leading ones. Best-performing DMUs are those finding an exact value of one for the inefficiency index. The comparison can lead us into setting benchmarks as a means to prioritize DMU-specific safety requirements. By doing so, a set of road safety measures are recommended which can best contribute the province to achieve the inefficiency score equaling one. Thereby, the concept of benchmarking is used as an applicatory approach in the literature of performance evaluation analysis. The approach identifies reference entities for each individual inefficient DMU on the basis of duality theory [20]. By such a process, a province in a given year can even be a benchmark for the same province but in the next year. Benchmarks for each inefficient unit are DMUs, the attributed constraints of which exactly equal zero in equation (10). In other words, benchmarks embody the constraints reflecting the DMUs with exactly equal weighted sum of inputs and weighted sum of outputs so that the corresponding inefficiency equals one. To do so, the concept of the dual price is utilized. Dual price or namely shadow price is the maximum price that management is willing to pay for an extra unit of a given limited resource [21]. The dual price attributed to each benchmark is earned by solving the dual model corresponding to the inefficient DMU under study. Table 3 illustrates an array of the variables and coefficients embodied in both main DEA and dual models.

Using the combinations in Table 2, the dual model is built up as following:

| (8) | Max ω0 |
| --- | --- |
|  | Subject to: |
| (9) | j=160yrjλj≤yr (r=1,2) |
| (10) | xiω0−j=160xijλj≤0 (i=1,…,6) |
|  | ω0,λj≥∘ |

In these equations ω0 is the objective value to be maximized in the dual model. The decision variable λj represents the dual price for jth DMU under study so that the values λj≠0 reveal the applicable dual prices for benchmark units. Finally, using the experiences by Hermans et al. [6], the target actions with the corresponding values for road safety performance indicators in inefficient provinces can be calculated as below:

Targeti,A=b=1BλbIIA×xi,b									(11)

where:

Targeti,A= target value for input data i in inefficient unit A

B= number of benchmarks for inefficient unit A

λb= dual price for benchmark b

IIA= inefficiency index for inefficient unit A

xi,b= existing value of input data i in benchmark b

Provided that the target is achieved, the inefficiency rate will be minimized as far as it equals 1. Having identified the targets, the needed changes in input values are determined. Notice that the targets and changes are not merely defined for the input values but the output target and changes are set for the fatality rates as well in the sense that the decrease in fatality rates can be estimated by such an analysis. In the next section, calculations are carried out for 30 provinces in two years with the results discussed in term of benchmarking effects.

5. Results and Discussion

This study covers the road safety performance data and the fatality risk indices available in all provinces of Iran (30 provinces) for two years (2008 and 2009). The solver add-in program in Microsoft Excel software was used as the tool to conduct the calculations for linear programming.

Having assigned the values ui(i=1,2) and vj(j=1,…,6) as the decision variables of each DMU, the problem is solved having found the minimized inefficiency values. Resulted inefficiency values are shown in Table 4. Notice that four provinces of Ardebil, Ilam, Booshehr, and Yazd in 2008 and eleven provinces of Ardebil, Isfahan, Booshehr, Tehran, Chaharmahal, Khorasan S., Semnan, Ghom, Kohgiluye, Hormozgan, and Yazd in year 2009 have found the least inefficiencies equal 1. Thereby, in 2009 a considerable advancement is observed in the number of efficient provinces compared with the year 2008. This is earned by lower fatalities and more road safety implementations in 2009 in comparison with year 2008. The average inefficiency value in 2008, has changed from 1.409 in 2008 to 1.307 in 2009 thereby a 7 percent reduction in inefficiency has occurred in the next year.

Given calculated inefficiency scores for each DMU, we can identify benchmarks for unsuccessful provinces and prioritize for road safety strategies afterwards. At the first step, the dual prices for benchmarks of a given DMU can be calculated once the benchmark identification is fulfilled. To do so, the zero valued constraints attributed to 60 DMUs are found as benchmarks whose corresponding dual prices are not equal to zero. The prices can be earned by solving the dual model specified for each DMU under study. The benchmarks and their attributed dual prices are shown in Table 3 for all 30 provinces in both years 2008 and 2009.

Using the dual prices, targets can be set for each road safety measure to be prioritized in a given unsuccessful province. Using equation 11, targets for a given DMU are estimated as the sum of the product of duals and the attributed benchmarks’ existing values totally divided by inefficiency index in the DMU under study. In addition to the input values, outputs can also be targeted by product of the duals and benchmarks’ existing output values. This is the approach which could be utilized in road fatality target setting efforts. Table 5 illustrates the resulted targets for all DMUs. Consider that the targets for successful DMUs are just in the same values as what currently exist.

Targets shown in Table 5 are the ones recommended for underperforming units in two years so that a comparison analysis can be made for the inefficiency rates to explore how changes occurred in two years. However, two restricting subjects and regarded solutions proposed to overcome can be deliberated here. The first is the concern about efficient units or benchmarks which have not found any unit to follow as a better performing one. Somehow, the benchmarks can also find a better situation in a special analytical context. The second issue relates to the two-year data comparison in the same province.

5.1. Efficient Units Analysis

Among the 30 provinces analyzed in two years, 4 provinces in 2008 and 11 provinces in 2009 are identified as the best-performing ones according to the least inefficiency rates as well as the benchmarking process shown in Table 4. Targets for all other 45 underperforming DMUs are adjusted to these 15 benchmarks but the question is that if the benchmarks themselves could follow a path to enhance the performances accordingly. The solution can be illustrated by defining a new special analytical context in which the benchmarks compete to achieve the lowest inefficiency rates in turn so that the ones disabled to do so are supposed to follow the new benchmarks generated in the context. For doing so, a new DEA process can be run for all 15 old benchmarks thereby the new inefficiency values are set to find new benchmarks. For the analysis of 15 new DMUs, the existing data set implies them again laying on the frontier so that all inefficiency rates will again equal to 1. On the other hand, a few inputs may be found which could have best helped them be the benchmarks in the initial analysis. Such a contribution can be shown by the individual shares of effects earned by product of input value and the attributed weight (vixi) for each DMU. Once the shares are estimated for all data in all 15 old benchmarks, the highest share show the most contributing factor thus could be eliminated in the new DEA process. That is because the most contributing factors are the main reasons for causing the mentioned DMUs to get efficient and their magnitudes are just in the convincing situation, thereby less contributing factors have not been located in a convincing level and they can be analyzed in an independent environment. Effective factors for whole analysis are determined by averaging the shares for all 15 DMUs thereby the overall shares for six input measures are 15% for PO, 9% for BS, 21% for H_F, 5% for SCC, 28% for EMS, and 22% for Li. The results show the EMS effects as the best contributing factor throughout the old benchmarks so that it can be removed by the analysis of competing benchmarks. Thereby, five inputs and two outputs so totally seven data types are remained for the analysis of 15 DMUs. The case would perfectly disorder the balance required to keep the inequality mentioned as the fourth prerequisite in selecting the indices in section 2 (15<3×7) so that much DMUs would still lie on the frontier line. Therefore, two more inputs are required to be removed from the analysis. The second and third high values of the shares belong to the road lighting effects by 22 percent and Highway_Freeway effects by 21 percent respectively, removing of which would keep the balance in data set although located on a border edge (15≥3×5). Running the DEA for total five data types makes some DMUs earning the least inefficiency value equaling one and still lying on the frontier line. In such a case, some other DMUs find higher inefficiency values. The results found for the analysis of 15 DMUs known as the old benchmarks with a three-input two-output data set is summarized in Table 6.

By new DEA analysis, seven new DMUs are identified as new benchmarks which represent the best performing ones. Similarly, targets can be set for those that could have not succeeded to appeal the lowest inefficiency rates in the new process. Recall that in the recent prioritization run, no targets thus no changes are subjected to set for EMS, lighting, and Highway_Freeway effects in the analysis. Since still there are seven DMUs remained as benchmarks, it might be possible to set a new analysis to rank them. But notice that seven DMUs need no more than 2 data types to keep the balance in data set (7≥3×2) in the sense that maximum one input and maximum one output would build up the data set in DEA analysis. As a rule of thumb, such an analysis will surely present far counterintuitive results so that we accept those seven DMUs as the ultimate ‘benchmarks of benchmarks’.

It must be noted that cross efficiency model (CEM) and super-efficiency method are two conventional methodologies introduced to rank efficient units but none of them can be applied here due to lack of compatibility with the scope of this study. The CEM method [22] ranks decision making units in lower efficiency (i.e. higher inefficiency) rates so that much less DMUs will earn the exact efficient position. Thus, a different set of road safety inefficiency values are estimated while disabled to identify the objective targets by benchmarking task. Moreover, by using the CEM approach the optimal input and output weights obtained from the basic DEA model may not be unique and this makes the cross-efficiency analysis somewhat arbitrary and limits its applicability [9]. Besides, super-efficiency (SE) method [23] ranks efficient DMUs below the lowest road safety inefficiency and permits them to pass by the efficient frontier and possess an inefficiency rate less than one. Inefficiency results by SE method are shown for efficient units in Table 6. Comparing the results by SE method with the new inefficiency rates show that the lowest SE results cope with new benchmarks which have again earned the rate equal to 1. Such a result confirms the approach developed here. Super-efficiency method doesn’t change the inefficiency values for under-performing units but is still disable to set targets for best-performing DMUs preferably in a competing context.

5.2. Two-Year Comparison

A variety of changing trends in two years can be observed in all provinces under study. The changes occurred in 2009 compared with 2008 are depicted in Fig 1. The columns show the decrease or increase in inefficiency values in percent.

The upper columns above the zero axes represent the provinces that have acted not in the desired direction since the inefficiency rates have grown up. Amongst them, the province Khorasan-N has met a great increase in inefficiency (more than 30 percent). On the contrary, the lower columns beneath the zero axes indicate the relatively good performing provinces which could have handled the inefficiency to some extent. Kurdistan is the most successful province among the relatively good performing ones.

For a further insight, Hormozgan is selected to be discussed as a province which has successfully reduced the inefficiency rate from a rather high value of 1.445 to the least value of 1. The column charts for the two-year implementations as well as the target values in the province are depicted in Fig. 2. According to the target values shown in dotted columns, the second-year actual RSPIs are expected to be equal to the targets while great distances are observed between the efficiently implemented and target values. Implemented measures in 2009 suggest a higher application of black spot remedial works and speed control cameras as well as less H_F rates. On the other side, target values advise higher rates of highway and freeways and lighting projects. Both implemented and target values would have led the province into the perfect efficient situation but in two different ways. Therefore, the proposed targets achieved by benchmarking process are not the unique solutions for the policy making problem. In the real world, several ideas may be given as managerial decisions based on the existing limited resources which may not make the targets earned in Table 4 as the practicable plans. To this end, utilizing a decision support system (DSS) can best help us test the outcomes achievable by several alternative decisions made to develop the road safety measures. To do so, providing a DEA-based knowledge base can help us establish the DSS by the means of a somehow expert system.

The DEA target set acts as a so called rigid framework of priorities that gives a unique set of information about the magnitude of road safety measures to be allocated as well as the effectiveness of the recommendations. In fact, the flexibility and programmability of strategies due to resource limitations is a critical issue of concern. An expert system in the light of a reasoning approach enables users to define alternative strategies by exact amount of measures to be assigned. Collecting a set of data in a few years as a knowledge base of the analysis can lead us into a decision making system, valid enough for all regions included in the study. Thus, one can use a single box of a decision making system (DMS) as a tool to be applied globally on the prospective years.

Not only the idea to build up a decision making system never conflicts the method used in this study, the DEA target setting can also significantly contribute the enhancements in such a system. Indeed, such a decision making system should include the targets set by the approach used in this study as the exact recommendations throughout a set of decision making units. By doing so, a comprehensive knowledge base can be constructed which can best propose alternative options with clear outcomes.

6. Conclusion

This study tries to build up a data envelopment analysis (DEA) process to evaluate the road safety performances experienced in 30 Iranian provinces for two years 2008 and 2009. Thereby, totally 60 decision making units (DMUs) are subjected to be used in the analysis. Six input values as the road safety measures or the so called road safety performance indicators are introduced in the analysis to be assessed versus two types of fatality rates used as the risk indices. Besides, amongst the 60 DMUs provided, a group of best performing provinces i.e. benchmarks are explored that can show the path to other underperforming ones. For doing so, the duality theory is applied by the means of which a set of ideal targets is found for each individual DMU. As a result, 15 DMUs are identified as benchmarks for the other ones so that 4 DMUs are adjusted in year 2008 and the other 11 ones in 2009. Benchmarks as the reference sets help DMUs seek for the targets that bring them the lowest inefficiency score. Targets are estimated by the means of dual prices as well as the measures of existing values in corresponding benchmarks. As observed by the target set, no changes are needed in the existing data values in benchmarks. Target setting and required changes are estimated both for input road safety performance indicators (i.e. increase) and output fatality rates (i.e. decrease). This study is an extended application of the method developed by Hermans et al. [6] but on a national basis carried out for all provinces in a single country in two successive years. The strategies made by such a process can be adopted either by the central governmental authorities or individually on a provincial basis. Discussion on results show that still some limitations exist to count in the adopted target setting approach as an inclusive decision making system due to the lack of alternatives that can be replaced by unique targets in the case of infeasibility.

Once the prioritization task is fulfilled, a new analytical context may be defined for 15 best performing DMUs in the sense that they are involved in a new competing ranking effort. To this end, the input data that have already contributed the lowest inefficiency achievement are eliminated in order to keep the balance in data set so that avoiding less DMUs to be located on the frontier. Considering the average product of input data and the attributed weights, defined as the shares to affect the benchmarks, three input data including EMS, lighting facilities, and freeway effects are removed in the new context. By doing so, eight DMUs amongst the old 15 efficient ones would be ranked as the new underperforming entities which can still enhance the efficiency rates regarding the new targets calculated for police operation, black spot treatments, and speed control cameras.

The other controversial discussion on results concerns the two-year comparison analysis. The results show that targets achieved by benchmarking process may not be the unique solutions to improve the efficiency, especially when constrained by some limited resources. For example, the targets set for Hormozgan province in 2008 are far different from the measures actually implemented in 2009 while both could have led into the lowest inefficiency rates. Thereby, some alternative practical decisions may be made in order to enhance the efficiencies which can be supported by a decision support system (DSS). Accordingly, feasible outcomes may be observed by developing a DEA-based knowledge base that can help us establish the DSS by the means of a somehow expert system. Indeed, such a system should include the targets set by the approach used in this study as the exact recommendations throughout a set of decision making units. Therefore, the DEA benchmarking and target setting tasks can significantly contribute the establishment of a comprehensive road safety decision making system.

References

- Iranian Road Safety Commission, “Main Performance Indicators toward Sustainable Road Safety, The Act by 31st Meeting of the Iranian Road Safety Commission”, Ministry of Road and Transportation, Iran (2009).

- Cook, W.D., Kazakov, A. and Persaud, B.N. “Prioritizing Highway Accident Sites: a Data Envelopment Analysis Model. Journal of the Operational Research Society 52, 303–309 (2001).

- Odeck, J. “Identifying traffic safety best practice: an application of DEA and Malmquist indices”, Omega 34, 28 – 40 (2006).

- Hermans, E., Van Den Bossche, F. and Wets, G. “Combining Road Safety Information in a Performance Index”, Accident Analysis and Prevention, 40 (2008) 1337-1344 (2008).

- Run-de, Y., Hong-bin, Z., Peng, S. and Fang, L. “The Study of DEA Evaluating Model on Road Traffic Safety”, World Congress on Engineering Vol. II, London, UK (2008).

- Hermans, E., Brijs, T., Wets, G. and Vanhoof, K. “Benchmarking Road Safety: Lessons to Learn from a Data Envelopment Analysis”. Accident Analysis and Prevention 41, 174-182 (2009).

- Sarkar, S. and Tay, R. “Prioritizing Road Safety Funding among Geographic Regions using Data Envelopment Analysis. 88th TRB Annual Meeting, Transportation Research Board, Washington D.C. (2009).

- Shen, Y., Hermans, E., Ruan, D., Wets, G., Brijs, T. and Vanhoof, K. “A generalized multiple layer data envelopment analysis model for hierarchical structure assessment: A case study in road safety performance evaluation”, Expert Systems with Applications 38, 15262–15272 (2011).

- Shen, Y., Hermans, E., Brijs, T., Wets, G. and Vanhoof, K. “Road safety risk evaluation and target setting using data envelopment analysis and its extensions”, Accident Analysis and Prevention, doi:10.1016/j.aap.2012.02.020 (2012).

- Adriaanse, A. “Environmental Policy Performance Indicators. A Study on the Development of Indicators for Environmental Policy in the Netherlands”, Ministry of Housing, Physical Planning and the Environment. Sdu Publishers, The Hague (1993).

- Wegman, F. and Oppe, S. “Benchmarking Road Safety Performances of Countries”, Safety Science, Volume 48, Issue 9, p.p. 1203–1211 (2010).

- European Transport Safety Council (ETSC). “Transport Safety Performance Indicators”, Brussels (2001).

- Mehregan, M., “Quantitative Models in Organizational Performance Evaluation (Data Envelopment Analysis)”, Faculty of Management Publications, University of Tehran, Second Edition (2006).

- Tingvall, C., Stigson, H., Eriksson, L., Johansson, R., Kraft, M. and Lie, A. “The Properties of Safety Performance Indicators in Target Setting, Projections and Safety Design of the Road Transport System”, Accident Analysis and Prevention 42 (4), 372-376 (2010).

- Management and Programming Organization of Iran (MPOI), “Road Pavement Rehabilitation Manual”, Code 296 (2004).

- Road Maintenance and Transportation Organization, “Highway Transportation Annual Reports”, Ministry of Road and Transportation, Iran (2009).

- Emrouznejad, A., Parker, B.R. and Tavares, G. “Evaluation of research in efficiency and productivity: A survey and analysis of the first 30 years of scholarly literature in DEA”. Socio-Economic Planning Sciences 42, 151–157 (2008).

- Cook, W.D. and Seiford, L.M. “Data envelopment analysis (DEA) – Thirty years on”, European Journal of Operational Research 192, 1–17 (2009).

- Charnes, A., Cooper, W.W. and Rohdes, E. “Measuring the Efficiency of Decision Making Units”, European Journal of Operational Research, 429-444 (1978).

- Hillier, F.S. and Lieberman, G.J. “Introduction to Operations Research, Seventh Edition”, McGraw-Hill Higher Education (2001).

- Siegel, J.G. and Shim, J.K. “Dictionary of Accounting Terms, Barron's business guides”, 5th Edition, Barron's snippet (2010).

- Sexton, T.R., Silkman, R.H. and Hogan, A.J. “Data envelopment analysis: critique and extensions”, In: Silkman, R.H. (Ed.), Measuring Efficiency: An Assessment of Data Envelopment Analysis. Jossey-Bass, San Francisco, pp. 73–104 (1986).

- Andersen, P. and Petersen, N.C. “A Procedure for Ranking Efficient Units in Data Envelopment Analysis”, Management Science, 39:1261-1264 (1993).

Authors’ Biography

Hamid Reza Behnood has received PhD in highway and transportation engineering in Ferdowsi University of Mashhad, Iran. His major studies were especially concentrated on road safety planning and economics using various techniques such as cost-benefit analysis, mathematical programming, statistical evaluations, data envelopment analysis, and fuzzy decision making. He had a significant role in establishment of the Techno-Economic Road Safety Research Center in Ferdowsi Univeristy of Mashhad.

Esmaeel Ayati is a Full Professor in the Department of Engineering at Ferdowsi University of Mashhad, Iran. He received his M.D. in Road and Civil Engineering from Tehran University and his Ph.D. in Road and Transportation Engineering from Aston University. His area of expertise is safety of traffic and his researches on evaluating the cost of traffic accidents is unique in Iran and his book titled “The Cost of Traffic Accidents in Iran” is the only valid reference in this area in Iran. He is the chief of Techno-Economic Road Safety Research Center in Ferdowsi Univeristy of Mashhad.

Elke Hermans graduated from the Limburg University Centre (now: Hasselt University) as Commercial Engineer in 2004. From 2004 until 2009 she worked as a doctoral researcher at the Transportation Research Institute of Hasselt University. Her research is situated in the area of road safety. More specifically, it focused on the development of indicators and a methodology for developing a composite road safety performance index that can be used for cross-country comparison. She attained her Ph.D. in 2009. Since then, she works as a professor at the Transportation Research Institute/Bachelor & Master Programme Transportation Sciences of Hasselt University. Elke Hermans is author and co-author of various journal articles (in Accident Analysis and Prevention, Transportation Research Record, Knowledge-Based Systems, etc) and conference proceedings (such as Annual Meeting of the Transportation Research Board, European Transport Conference, International Road Traffic and Accident Database Conference, International Conference on Sensitivity Analysis of Model Output, etc).

Mohammadali Pirayesh Neghab received his B.Sc., M.Sc. & Ph.D. degrees in Industrial Engineering from Sharif University of Technology in 2000, 2002 and 2008, respectively. Currently he is a faculty member in the Industrial Engineering Department at Ferdowsi University of Mashhad. His current research interests include supply chain management, transportation planning and fuzzy decision making.

Table 1. DEA input and output data

| Year | Province | DMU | PO | BS | H_F | SCC | EMS | Li | FR1=F/VKT | FR2=F/L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2008 | Azerbaijan-E | 1 | 0.32 | 1.90 | 27.79 | 0.00 | 1.45 | 2.68 | 0.805 | 16.6 |
|  | Azerbaijan-W | 2 | 0.26 | 1.39 | 7.53 | 0.00 | 0.84 | 2.67 | 1.228 | 18.2 |
|  | Ardebil | 3 | 0.37 | 1.05 | 11.01 | 0.00 | 1.57 | 9.86 | 0.980 | 12.7 |
|  | Isfahan | 4 | 0.28 | 1.47 | 60.57 | 1.62 | 1.44 | 2.13 | 0.587 | 16.9 |
|  | Ilam | 5 | 0.28 | 0.35 | 3.05 | 0.00 | 1.40 | 0.14 | 1.531 | 8.9 |
|  | Booshehr | 6 | 0.30 | 0.66 | 43.33 | 0.00 | 1.79 | 2.80 | 0.697 | 11.3 |
|  | Tehran | 7 | 1.06 | 0.76 | 83.40 | 9.80 | 5.32 | 18.98 | 0.430 | 70.0 |
|  | Chaharmahal | 8 | 0.28 | 1.59 | 4.28 | 0.00 | 1.59 | 3.53 | 1.519 | 10.8 |
|  | Khorasan-S | 9 | 0.10 | 0.18 | 0.66 | 0.00 | 0.86 | 1.40 | 1.409 | 4.7 |
|  | Khorasan-Raz | 10 | 0.29 | 1.03 | 24.48 | 0.00 | 1.59 | 2.78 | 0.772 | 16.9 |
|  | Khorasan-N | 11 | 0.36 | 1.25 | 12.84 | 0.00 | 1.16 | 4.55 | 1.823 | 16.2 |
|  | Khoozestan | 12 | 0.23 | 1.65 | 31.23 | 0.00 | 1.74 | 1.67 | 0.518 | 17.6 |
|  | Zanjan | 13 | 0.43 | 2.80 | 41.83 | 0.00 | 2.01 | 4.24 | 2.130 | 21.9 |
|  | Semnan | 14 | 0.54 | 1.14 | 60.59 | 0.00 | 2.08 | 5.65 | 1.176 | 22.5 |
|  | Sistan | 15 | 0.16 | 0.41 | 0.66 | 0.00 | 1.08 | 0.45 | 1.376 | 11.2 |
|  | Fars | 16 | 0.23 | 2.01 | 14.70 | 0.00 | 1.48 | 1.85 | 1.206 | 17.6 |
|  | Ghazvin | 17 | 0.49 | 3.13 | 65.27 | 6.43 | 1.98 | 8.08 | 2.005 | 35.8 |
|  | Ghom | 18 | 0.67 | 3.03 | 102.77 | 12.73 | 2.69 | 14.12 | 1.335 | 41.8 |
|  | Kurdistan | 19 | 0.25 | 2.46 | 4.70 | 0.00 | 1.20 | 2.84 | 2.063 | 25.4 |
|  | Kerman | 20 | 0.20 | 2.50 | 19.97 | 0.00 | 1.54 | 0.72 | 0.960 | 13.2 |
|  | Kermanshah | 21 | 0.38 | 0.63 | 18.50 | 0.00 | 0.79 | 5.60 | 1.367 | 14.9 |
|  | Kohgiluyeh | 22 | 0.27 | 0.82 | 2.33 | 0.00 | 2.01 | 2.19 | 2.337 | 10.1 |
|  | Golestan | 23 | 0.36 | 2.63 | 21.43 | 0.00 | 2.63 | 13.95 | 1.569 | 31.9 |
|  | Gilan | 24 | 0.48 | 2.55 | 30.63 | 0.00 | 1.78 | 7.96 | 1.896 | 37.3 |
|  | Lorestan | 25 | 0.43 | 1.66 | 15.65 | 0.00 | 2.96 | 4.99 | 2.120 | 30.1 |
|  | Mazandaran | 26 | 0.49 | 3.26 | 31.22 | 0.00 | 2.24 | 11.19 | 1.325 | 28.5 |
|  | Markazi | 27 | 0.42 | 1.48 | 34.76 | 2.89 | 1.37 | 3.96 | 0.986 | 19.4 |
|  | Hormozgan | 28 | 0.17 | 0.20 | 14.92 | 0.00 | 0.99 | 0.72 | 0.265 | 12.2 |
|  | Hamedan | 29 | 0.30 | 1.14 | 44.14 | 0.00 | 1.50 | 2.58 | 1.650 | 28.2 |
|  | Yazd | 30 | 0.21 | 0.31 | 19.63 | 0.00 | 0.82 | 1.47 | 0.433 | 7.6 |
| Mean |  |  | 0.35 | 1.51 | 28.46 | 1.12 | 1.73 | 4.86 | 1.283 | 21.0 |
| 2009 | Azerbaijan-E | 31 | 0.32 | 1.23 | 27.93 | 0.00 | 1.65 | 3.85 | 0.771 | 16.7 |
|  | Azerbaijan-W | 32 | 0.26 | 3.18 | 8.83 | 0.00 | 1.20 | 2.66 | 1.197 | 18.0 |
|  | Ardebil | 33 | 0.37 | 7.32 | 12.78 | 0.00 | 1.64 | 10.23 | 1.031 | 14.1 |
|  | Isfahan | 34 | 0.28 | 1.54 | 72.60 | 1.25 | 1.46 | 2.93 | 0.547 | 16.8 |
|  | Ilam | 35 | 0.28 | 2.71 | 3.29 | 0.00 | 1.39 | 0.76 | 1.921 | 11.5 |
|  | Booshehr | 36 | 0.29 | 2.92 | 48.96 | 0.65 | 1.75 | 4.09 | 0.669 | 12.7 |
|  | Tehran | 37 | 1.05 | 3.84 | 88.29 | 8.66 | 5.79 | 20.92 | 0.338 | 60.0 |
|  | Chaharmahal | 38 | 0.26 | 2.07 | 9.42 | 0.00 | 1.81 | 3.36 | 1.209 | 9.3 |
|  | Khorasan-S | 39 | 0.10 | 0.31 | 0.90 | 0.00 | 0.96 | 1.40 | 1.456 | 4.7 |
|  | Khorasan-Raz | 40 | 0.28 | 0.93 | 26.26 | 0.00 | 1.59 | 2.84 | 0.750 | 17.1 |
|  | Khorasan-N | 41 | 0.36 | 3.57 | 15.78 | 0.00 | 1.34 | 4.83 | 2.210 | 23.0 |
|  | Khoozestan | 42 | 0.23 | 3.49 | 32.55 | 0.00 | 1.78 | 3.26 | 0.532 | 17.4 |
|  | Zanjan | 43 | 0.43 | 2.95 | 43.92 | 4.29 | 2.30 | 4.24 | 1.971 | 21.4 |
|  | Semnan | 44 | 0.60 | 2.78 | 76.71 | 0.00 | 2.63 | 7.51 | 1.152 | 24.1 |
|  | Sistan | 45 | 0.16 | 1.10 | 3.76 | 0.00 | 1.19 | 0.86 | 1.391 | 11.4 |
|  | Fars | 46 | 0.22 | 2.02 | 19.14 | 0.00 | 1.50 | 1.97 | 1.120 | 17.1 |
|  | Ghazvin | 47 | 0.48 | 3.14 | 72.66 | 8.27 | 2.25 | 13.12 | 1.875 | 35.1 |
|  | Ghom | 48 | 0.65 | 2.94 | 104.97 | 12.68 | 2.61 | 17.16 | 1.344 | 42.5 |
|  | Kurdistan | 49 | 0.25 | 5.23 | 8.49 | 0.00 | 1.64 | 2.84 | 1.771 | 22.4 |
|  | Kerman | 50 | 0.19 | 1.79 | 24.68 | 0.94 | 1.60 | 1.79 | 0.853 | 12.2 |
|  | Kermanshah | 51 | 0.37 | 1.99 | 18.63 | 1.21 | 1.04 | 6.01 | 1.500 | 16.1 |
|  | Kohgiluyeh | 52 | 0.27 | 1.83 | 4.66 | 0.00 | 2.01 | 2.56 | 2.015 | 9.6 |
|  | Golestan | 53 | 0.36 | 1.27 | 29.01 | 0.00 | 2.54 | 13.95 | 1.466 | 29.8 |
|  | Gilan | 54 | 0.47 | 2.91 | 36.76 | 0.00 | 1.81 | 8.04 | 1.655 | 36.5 |
|  | Lorestan | 55 | 0.43 | 7.64 | 19.76 | 0.00 | 3.21 | 6.21 | 2.460 | 32.8 |
|  | Mazandaran | 56 | 0.48 | 3.91 | 33.80 | 0.00 | 2.56 | 12.07 | 1.464 | 32.3 |
|  | Markazi | 57 | 0.41 | 3.68 | 45.92 | 3.64 | 1.38 | 4.75 | 1.138 | 24.3 |
|  | Hormozgan | 58 | 0.17 | 4.81 | 23.23 | 4.37 | 0.99 | 0.78 | 0.263 | 12.1 |
|  | Hamedan | 59 | 0.30 | 5.42 | 43.80 | 0.00 | 1.67 | 3.87 | 1.612 | 28.6 |
|  | Yazd | 60 | 0.20 | 0.57 | 23.76 | 2.56 | 1.21 | 2.31 | 0.422 | 7.5 |
| Mean |  |  | 0.35 | 2.97 | 32.71 | 1.62 | 1.88 | 5.71 | 1.270 | 21.2 |

Table 2. Co-linearity check for input data (R2)

| Indicator | PO | BS | H_F | SCC | EMS | Li |
| --- | --- | --- | --- | --- | --- | --- |
| PO | 1.00 |  |  |  |  |  |
| BS | 0.05 | 1.00 |  |  |  |  |
| H_F | 0.53 | 0.03 | 1.00 |  |  |  |
| SCC | 0.44 | 0.02 | 0.59 | 1.00 |  |  |
| EMS | 0.74 | 0.07 | 0.34 | 0.29 | 1.00 |  |
| Li | 0.73 | 0.08 | 0.41 | 0.41 | 0.60 | 1.00 |

Table 3. Variables and coefficients to build up the dual model

| Right side value | v6 | ⋯ | v2 | v1 | u2 | u1 | Main model Dual model |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | −x61 | ⋯ | −x21 | −x11 | y21 | y11 | λ1 | Const. 1 |
| 0 | −x62 | ⋯ | −x22 | −x12 | y22 | y12 | λ2 | Const. 2 |
| ⋮ | ⋮ | ⋯ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ |
| 0 | −x6j | ⋯ | −x2j | −x1j | y2j | y1j | λj | Const. j |
| ⋮ | ⋮ | ⋯ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ | ⋮ |
| 0 | −x660 | ⋯ | −x260 | −x160 | y260 | y160 | λ60 | Const. 60 |
| =1 | x6 | ⋯ | x2 | x1 | 0 | 0 | ω0 | Const. 61 |
|  | 0 | ⋯ | 0 | 0 | y2 | y1 | Right side value |  |

Table 4. Benchmarking and dual price calculations

| Year | Province | DMU |  | Benchmarks (#DMU, Province, Year) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Inefficiency | 3 ARD (08) | 5 ILM (08) | 6 BOO (08) | 30 YZD (08) | 33 ARD (09) | 34 ISF (09) |  | 36 BOO (09) | 37 TEH (09) | 38 CHA (09) | 39 KHS (09) | 44 SEM (09) | 48 GHM (09) | 52 KOH (09) | 58 HOR (09) | 60 YZD (09) |
| 2008 | Azerbaijan-E | 1 | 1.279 |  |  |  |  | 0.19 |  |  |  | 0.06 |  |  |  |  |  |  | 1.39 |
|  | Azerbaijan-W | 2 | 1.974 | 0.58 |  |  |  | 0.21 |  |  |  |  |  |  |  |  |  |  | 1.04 |
|  | Ardebil | 3 | 1.000 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Isfahan | 4 | 1.105 |  |  |  |  |  | 0.76 |  | 0.07 | 0.02 |  |  | 0.02 |  |  |  | 0.25 |
|  | Ilam | 5 | 1.000 |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Booshehr | 6 | 1.000 |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Tehran | 7 | 1.079 |  |  |  |  |  |  |  |  | 1.13 |  |  |  |  |  | 0.18 |  |
|  | Chaharmahal | 8 | 1.131 | 0.17 | 0.3 |  |  | 0.07 |  |  |  |  | 0.3 |  |  |  | 0.23 |  |  |
|  | Khorasan-S | 9 | 1.088 |  |  |  |  |  |  |  |  |  | 0.18 | 0.63 |  |  |  |  |  |
|  | Khorasan-Raz | 10 | 1.452 |  |  |  |  | 0.04 |  |  |  | 0.06 |  |  |  |  |  |  | 1.68 |
|  | Khorasan-N | 11 | 1.341 | 0.41 | 0.75 | 0.01 |  | 0.11 |  |  |  |  |  |  |  |  |  |  | 0.37 |
|  | Khoozestan | 12 | 1.236 |  |  |  |  |  |  |  |  | 0.13 |  |  |  |  |  | 0.2 | 1 |
|  | Zanjan | 13 | 1.369 |  | 0.31 | 1.18 |  | 0.4 |  |  |  |  |  |  |  |  |  |  |  |
|  | Semnan | 14 | 1.093 |  |  |  | 0.47 |  |  |  |  | 0.03 |  |  |  |  |  |  | 2.28 |
|  | Sistan | 15 | 1.933 |  | 0.11 |  |  |  |  |  |  |  | 0.92 |  |  |  |  |  | 0.22 |
|  | Fars | 16 | 1.819 |  |  |  |  | 0.33 |  |  |  |  | 0.2 |  |  |  |  |  | 1.5 |
|  | Ghazvin | 17 | 1.590 |  |  |  |  | 0.29 |  |  | 0.16 |  |  |  |  | 0.05 |  | 0.58 | 3.61 |
|  | Ghom | 18 | 1.004 |  |  |  |  |  | 0.05 |  |  | 0.01 |  |  |  | 0.73 |  | 0.53 | 0.46 |
|  | Kurdistan | 19 | 2.809 | 0.83 |  |  |  | 0.71 |  |  |  |  | 0.36 |  |  |  |  |  | 0.21 |
|  | Kerman | 20 | 1.293 |  |  |  |  | 0.31 |  |  |  |  | 0.21 |  |  |  |  |  | 0.91 |
|  | Kermanshah | 21 | 1.151 | 0.57 | 0.37 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.58 |
|  | Kohgiluyeh | 22 | 1.052 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.05 |  |  |
|  | Golestan | 23 | 1.375 | 1.29 |  |  |  | 0.2 |  |  |  | 0.2 |  |  |  |  |  |  | 0.07 |
|  | Gilan | 24 | 1.980 | 0.3 |  |  |  | 0.37 |  |  |  | 0.12 |  |  |  |  |  |  | 2.8 |
|  | Lorestan | 25 | 1.702 |  |  |  |  |  |  |  |  |  | 0.62 |  |  |  |  |  | 3.25 |
|  | Mazandaran | 26 | 1.389 | 0.76 |  |  |  | 0.29 |  |  | 0.3 | 0.18 |  |  | 0.01 |  |  |  |  |
|  | Markazi | 27 | 1.184 |  |  |  |  | 0.05 |  |  |  | 0.04 |  |  |  |  |  |  | 2.19 |
|  | Hormozgan | 28 | 1.445 |  |  |  |  |  |  |  |  | 0.14 |  |  |  |  |  |  | 0.52 |
|  | Hamedan | 29 | 2.402 |  |  | 1.73 |  |  |  |  | 0.44 |  |  |  | 0.08 |  |  |  | 0.11 |
|  | Yazd | 30 | 1.000 |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2009 | Azerbaijan-E | 31 | 1.284 |  |  |  |  | 0.06 |  |  |  | 0.06 |  |  |  |  |  |  | 1.64 |
|  | Azerbaijan-W | 32 | 1.878 | 0.04 |  |  |  | 0.74 |  |  |  |  |  |  |  |  |  |  | 0.95 |
|  | Ardebil | 33 | 1.000 |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |
|  | Isfahan | 34 | 1.000 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |
|  | Ilam | 35 | 1.195 |  | 0.55 |  |  | 0.38 |  |  |  |  |  |  |  |  | 0.13 |  |  |
|  | Booshehr | 36 | 1.000 |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
|  | Tehran | 37 | 1.000 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
|  | Chaharmahal | 38 | 1.000 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |
|  | Khorasan-S | 39 | 1.000 |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
|  | Khorasan-Raz | 40 | 1.486 |  |  |  |  | 0.02 |  |  |  | 0.07 |  |  |  |  |  |  | 1.67 |
|  | Khorasan-N | 41 | 1.800 | 0.07 | 0.69 |  |  | 0.79 |  |  |  |  |  |  |  |  |  |  | 0.65 |
|  | Khoozestan | 42 | 1.096 |  |  |  |  |  |  |  | 0.01 | 0.06 |  |  |  |  |  | 0.65 | 0.8 |
|  | Zanjan | 43 | 1.313 |  |  |  |  | 0.32 |  |  | 0.05 |  |  |  |  |  |  | 0.04 | 2.12 |
|  | Semnan | 44 | 1.000 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
|  | Sistan | 45 | 1.830 |  |  |  |  |  |  |  |  |  | 1.1 |  |  |  |  |  | 0.15 |
|  | Fars | 46 | 1.723 |  |  |  |  | 0.32 |  |  |  |  | 0.11 |  |  |  |  |  | 1.54 |
|  | Ghazvin | 47 | 1.150 | 0.1 |  |  |  | 0.18 |  |  |  |  |  |  |  | 0.64 |  |  | 0.55 |
|  | Ghom | 48 | 1.000 |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |
|  | Kurdistan | 49 | 1.814 |  |  |  |  | 1.17 |  |  |  |  | 0.34 |  |  |  |  |  | 0.36 |
|  | Kerman | 50 | 1.208 |  |  |  |  | 0.16 |  |  |  |  | 0.18 |  |  |  |  |  | 1.12 |
|  | Kermanshah | 51 | 1.236 | 0.38 | 0.41 | 0.02 |  | 0.21 |  |  |  |  |  |  |  |  |  |  | 0.59 |
|  | Kohgiluyeh | 52 | 1.000 |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
|  | Golestan | 53 | 1.276 | 1.33 |  |  |  |  |  |  | 0.01 | 0.16 |  |  |  | 0.07 |  |  |  |
|  | Gilan | 54 | 1.893 | 0.14 |  |  |  | 0.46 |  |  |  | 0.18 |  |  |  |  |  |  | 2.34 |
|  | Lorestan | 55 | 1.441 |  |  |  |  | 1.26 |  |  |  |  | 0.46 |  |  |  |  |  | 1.43 |
|  | Mazandaran | 56 | 1.439 | 0.73 |  |  |  | 0.43 |  |  | 0.31 | 0.21 |  |  |  |  |  |  | 0.07 |
|  | Markazi | 57 | 1.380 |  |  |  |  | 0.25 |  |  | 0.4 | 0.06 |  |  |  |  |  | 0.24 | 1.26 |
|  | Hormozgan | 58 | 1.000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
|  | Hamedan | 59 | 1.783 |  |  |  |  | 0.65 |  |  | 1.32 |  |  |  |  |  |  | 0.21 |  |
|  | Yazd | 60 | 1.000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |

Table 5. Target setting in all DMUs

| Year | Province | DMU | Ineff | Target Values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PO | BS | H_F | SC | EMS | Li | FR1 | FR2 |
| 2008 | Azerbaijan-E | 1 | 1.279 | 0.323 | 1.885 | 31.860 | 3.192 | 1.825 | 5.011 | 0.627 | 13.022 |
|  | Azerbaijan-W | 2 | 1.974 | 0.255 | 1.386 | 17.117 | 1.351 | 1.271 | 5.203 | 0.620 | 9.167 |
|  | Ardebil | 3 | 1.000 | 0.373 | 1.046 | 11.010 | 0.000 | 1.568 | 9.858 | 0.980 | 12.696 |
|  | Isfahan | 4 | 1.105 | 0.286 | 1.495 | 61.426 | 1.641 | 1.539 | 3.316 | 0.541 | 15.586 |
|  | Ilam | 5 | 1.000 | 0.279 | 0.349 | 3.050 | 0.000 | 1.397 | 0.140 | 1.531 | 8.869 |
|  | Booshehr | 6 | 1.000 | 0.298 | 0.656 | 43.328 | 0.000 | 1.790 | 2.804 | 0.697 | 11.337 |
|  | Tehran | 7 | 1.079 | 1.132 | 4.822 | 96.345 | 9.795 | 6.233 | 22.040 | 0.398 | 64.837 |
|  | Chaharmahal | 8 | 1.131 | 0.278 | 1.624 | 6.702 | 0.000 | 1.598 | 3.566 | 1.348 | 9.560 |
|  | Khorasan-S | 9 | 1.088 | 0.103 | 0.523 | 2.078 | 0.000 | 0.856 | 1.368 | 1.044 | 4.247 |
|  | Khorasan-Raz | 10 | 1.452 | 0.286 | 1.018 | 31.487 | 3.324 | 1.679 | 3.818 | 0.530 | 11.502 |
|  | Khorasan-N | 11 | 1.341 | 0.359 | 1.277 | 12.998 | 0.707 | 1.741 | 4.589 | 1.362 | 12.142 |
|  | Khoozestan | 12 | 1.236 | 0.301 | 1.643 | 32.278 | 3.693 | 1.745 | 4.197 | 0.419 | 14.316 |
|  | Zanjan | 13 | 1.369 | 0.430 | 2.784 | 41.784 | 0.000 | 2.340 | 5.440 | 1.249 | 15.909 |
|  | Semnan | 14 | 1.093 | 0.536 | 1.424 | 60.407 | 5.584 | 3.025 | 6.023 | 1.074 | 20.471 |
|  | Sistan | 15 | 1.933 | 0.162 | 1.070 | 7.361 | 0.292 | 1.079 | 1.872 | 0.711 | 5.788 |
|  | Fars | 16 | 1.819 | 0.262 | 2.025 | 22.949 | 2.114 | 1.491 | 4.131 | 0.668 | 9.741 |
|  | Ghazvin | 17 | 1.590 | 0.636 | 4.768 | 72.986 | 7.881 | 3.656 | 8.349 | 1.351 | 26.568 |
|  | Ghom | 18 | 1.004 | 0.682 | 5.052 | 103.949 | 12.845 | 3.105 | 14.299 | 1.339 | 42.147 |
|  | Kurdistan | 19 | 2.809 | 0.253 | 2.467 | 9.467 | 0.192 | 1.201 | 6.103 | 0.737 | 9.072 |
|  | Kerman | 20 | 1.293 | 0.273 | 2.492 | 21.318 | 1.805 | 1.536 | 4.625 | 0.740 | 10.152 |
|  | Kermanshah | 21 | 1.151 | 0.376 | 0.917 | 18.411 | 1.292 | 1.833 | 6.092 | 1.190 | 12.903 |
|  | Kohgiluyeh | 22 | 1.052 | 0.274 | 1.824 | 4.648 | 0.000 | 2.006 | 2.554 | 2.011 | 9.576 |
|  | Golestan | 23 | 1.375 | 0.568 | 2.633 | 26.246 | 1.390 | 2.615 | 13.901 | 1.140 | 23.073 |
|  | Gilan | 24 | 1.980 | 0.474 | 2.564 | 43.014 | 4.151 | 2.600 | 7.941 | 0.958 | 18.755 |
|  | Lorestan | 25 | 1.702 | 0.478 | 1.840 | 48.799 | 4.895 | 2.961 | 5.635 | 1.245 | 17.646 |
|  | Mazandaran | 26 | 1.389 | 0.486 | 3.248 | 31.252 | 1.262 | 2.349 | 11.177 | 0.948 | 20.575 |
|  | Markazi | 27 | 1.184 | 0.423 | 1.491 | 47.463 | 5.034 | 2.494 | 5.410 | 0.834 | 16.427 |
|  | Hormozgan | 28 | 1.445 | 0.174 | 0.577 | 17.103 | 1.761 | 0.995 | 2.858 | 0.184 | 8.496 |
|  | Hamedan | 29 | 2.402 | 0.298 | 1.127 | 43.826 | 0.237 | 1.754 | 3.126 | 0.682 | 11.637 |
|  | Yazd | 30 | 1.000 | 0.205 | 0.308 | 19.627 | 0.000 | 0.822 | 1.472 | 0.433 | 7.583 |
| 2009 | Azerbaijan-E | 31 | 1.284 | 0.323 | 1.248 | 35.064 | 3.678 | 1.886 | 4.405 | 0.602 | 12.994 |
|  | Azerbaijan-W | 32 | 1.878 | 0.257 | 3.194 | 17.292 | 1.297 | 1.291 | 5.411 | 0.640 | 9.610 |
|  | Ardebil | 33 | 1.000 | 0.373 | 7.319 | 12.777 | 0.000 | 1.643 | 10.232 | 1.031 | 14.115 |
|  | Isfahan | 34 | 1.000 | 0.278 | 1.542 | 72.602 | 1.253 | 1.456 | 2.934 | 0.547 | 16.813 |
|  | Ilam | 35 | 1.195 | 0.277 | 2.686 | 5.971 | 0.000 | 1.384 | 3.595 | 1.251 | 9.611 |
|  | Booshehr | 36 | 1.000 | 0.292 | 2.924 | 48.959 | 0.652 | 1.754 | 4.094 | 0.669 | 12.690 |
|  | Tehran | 37 | 1.000 | 1.053 | 3.837 | 88.287 | 8.656 | 5.794 | 20.918 | 0.338 | 59.970 |
|  | Chaharmahal | 38 | 1.000 | 0.259 | 2.070 | 9.418 | 0.000 | 1.811 | 3.364 | 1.209 | 9.314 |
|  | Khorasan-S | 39 | 1.000 | 0.104 | 0.311 | 0.897 | 0.000 | 0.960 | 1.401 | 1.456 | 4.670 |
|  | Khorasan-Raz | 40 | 1.486 | 0.280 | 0.919 | 31.036 | 3.289 | 1.650 | 3.719 | 0.504 | 11.405 |
|  | Khorasan-N | 41 | 1.800 | 0.358 | 3.593 | 15.789 | 0.926 | 1.753 | 5.763 | 1.230 | 12.787 |
|  | Khoozestan | 42 | 1.096 | 0.308 | 3.503 | 36.386 | 4.942 | 1.799 | 3.333 | 0.488 | 16.043 |
|  | Zanjan | 43 | 1.313 | 0.432 | 2.960 | 44.045 | 4.297 | 2.443 | 6.402 | 0.965 | 16.344 |
|  | Semnan | 44 | 1.000 | 0.601 | 2.780 | 76.711 | 0.000 | 2.630 | 7.513 | 1.152 | 24.117 |
|  | Sistan | 45 | 1.830 | 0.172 | 1.291 | 7.609 | 0.210 | 1.188 | 2.211 | 0.761 | 6.211 |
|  | Fars | 46 | 1.723 | 0.265 | 2.000 | 24.215 | 2.292 | 1.498 | 4.180 | 0.646 | 9.890 |
|  | Ghazvin | 47 | 1.150 | 0.551 | 3.145 | 72.731 | 8.280 | 2.425 | 13.110 | 1.196 | 30.523 |
|  | Ghom | 48 | 1.000 | 0.654 | 2.941 | 104.972 | 12.676 | 2.614 | 17.157 | 1.344 | 42.484 |
|  | Kurdistan | 49 | 1.814 | 0.329 | 5.223 | 14.725 | 0.509 | 1.639 | 7.690 | 0.976 | 12.334 |
|  | Kerman | 50 | 1.208 | 0.274 | 1.806 | 25.128 | 2.377 | 1.605 | 3.998 | 0.708 | 10.179 |
|  | Kermanshah | 51 | 1.236 | 0.372 | 1.963 | 18.609 | 1.224 | 1.829 | 5.963 | 1.197 | 12.989 |
|  | Kohgiluyeh | 52 | 1.000 | 0.274 | 1.828 | 4.658 | 0.000 | 2.011 | 2.559 | 2.015 | 9.598 |
|  | Golestan | 53 | 1.276 | 0.559 | 1.755 | 28.687 | 1.786 | 2.518 | 13.871 | 1.142 | 23.182 |
|  | Gilan | 54 | 1.893 | 0.444 | 1.805 | 39.393 | 3.994 | 2.495 | 5.761 | 1.093 | 18.680 |
|  | Lorestan | 55 | 1.441 | 0.609 | 7.627 | 37.770 | 2.545 | 3.212 | 12.316 | 1.706 | 22.730 |
|  | Mazandaran | 56 | 1.439 | 0.528 | 3.935 | 33.991 | 1.528 | 2.569 | 12.105 | 1.019 | 22.508 |
|  | Markazi | 57 | 1.380 | 0.411 | 3.696 | 46.075 | 3.666 | 2.330 | 6.194 | 0.826 | 17.769 |
|  | Hormozgan | 58 | 1.000 | 0.171 | 4.809 | 23.226 | 4.372 | 0.989 | 0.784 | 0.263 | 12.142 |
|  | Hamedan | 59 | 1.783 | 0.373 | 5.400 | 43.644 | 0.998 | 2.015 | 6.854 | 0.902 | 15.972 |
|  | Yazd | 60 | 1.000 | 0.201 | 0.569 | 23.763 | 2.564 | 1.205 | 2.310 | 0.422 | 7.466 |

Table 6. Benchmarking, dual prices and target setting for old benchmarks

| Year | Province | Old DMU | Ineff. by SE Method | New DMU | New Ineff. Rates | New Benchmarks (#New DMU, Province, Year) |  |  |  |  |  |  | New Target Values 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 1 ARD (08) | 2 ILM (08) | 4 YZD (08) | 5 ARD (09) | 8 TEH (09) | 14 HOR (09) | 15 YZD (09) | PO | BS | SCC | FR1 | FR2 |
| 2008 | Ardebil | 3 | 0.905 | 1 | 1.000 | 1.000 |  |  |  |  |  |  | 0.37 | 1.05 | 0.00 | 0.98 | 12.70 |
|  | Ilam | 5 | 0.927 | 2 | 1.000 |  | 1.000 |  |  |  |  |  | 0.28 | 0.35 | 0.00 | 1.53 | 8.87 |
|  | Booshehr | 6 | 0.936 | 3 | 1.048 | 0.204 |  | 0.713 |  |  |  | 0.447 | 0.30 | 0.66 | 1.09 | 0.67 | 10.82 |
|  | Yazd | 30 | 0.996 | 4 | 1.000 |  |  | 1.000 |  |  |  |  | 0.21 | 0.31 | 0.00 | 0.43 | 7.58 |
| 2009 | Ardebil | 33 | 0.507 | 5 | 1.000 |  |  |  | 1.000 |  |  |  | 0.37 | 7.32 | 0.00 | 1.03 | 14.12 |
|  | Isfahan | 34 | 0.793 | 6 | 1.334 |  |  |  | 0.141 | 0.143 |  | 0.838 | 0.28 | 1.54 | 2.54 | 0.41 | 12.61 |
|  | Booshehr | 36 | 0.843 | 7 | 1.078 |  |  |  | 0.355 | 0.043 |  | 0.685 | 0.29 | 2.92 | 1.97 | 0.62 | 11.78 |
|  | Tehran | 37 | 0.539 | 8 | 1.000 |  |  |  |  | 1.000 |  |  | 1.05 | 3.84 | 8.66 | 0.34 | 59.97 |
|  | Chaharmahal | 38 | 0.906 | 9 | 1.058 | 0.018 | 0.597 |  | 0.268 |  |  |  | 0.26 | 2.07 | 0.00 | 1.14 | 8.79 |
|  | Khorasan-S | 39 | 0.995 | 10 | 1.392 |  | 0.468 |  | 0.037 |  |  |  | 0.10 | 0.31 | 0.00 | 0.54 | 3.36 |
|  | Semnan | 44 | 0.980 | 11 | 1.002 |  |  |  | 0.161 | 0.081 |  | 2.274 | 0.60 | 2.78 | 6.52 | 1.15 | 24.05 |
|  | Ghom | 48 | 0.903 | 12 | 1.170 |  |  |  |  | 0.020 | 2.353 | 1.703 | 0.65 | 10.57 | 12.68 | 1.15 | 36.32 |
|  | Kohgiluyeh | 52 | 0.939 | 13 | 1.044 |  | 0.722 |  | 0.226 |  |  |  | 0.27 | 1.83 | 0.00 | 1.28 | 9.19 |
|  | Hormozgan | 58 | 0.381 | 14 | 1.000 |  |  |  |  |  | 1.000 |  | 0.17 | 4.81 | 4.37 | 0.26 | 12.14 |
|  | Yazd | 60 | 0.677 | 15 | 1.000 |  |  |  |  |  |  | 1.000 | 0.20 | 0.57 | 2.56 | 0.42 | 7.47 |

1. Targets for new efficient units are equal to the existing values so no changes are needed in related safety measures.

Fig. 1. Percent of changes occurred in inefficiency rates in 2009 compared with 2008

Fig. 2. RSPIs and Target Values for Hormozgan province