Study of CuFeO2 for photoelectrochemical CO2 conversion

Nele Debusschere1,3 Bjorn Joos1,2,3, Ken Elen1,2,3, Jan D’Haen4, Pegie Cool5, Marlies K. Van Bael1,2,3, An Hardy1,2,3

1 Hasselt Univeristy, Institute for Materials Research (imo-imomec), Design and Synthesis of Inorganic Materials (DESINe), Agoralaan Building D, B-3590 Diepenbeek, Belgium.

2 imo, imo-imomec, Wetenschapspark 1, B-3590 Diepenbeek, Belgium.

3 EnergyVille, Thor Park 8320, B-3600 Genk, Belgium.

4 Hasselt University, Institute for Materials Research (imo-imomec), Analytical & Microscopical Services (AMS), Wetenschapspark 1, 3590 Diepenbeek, Belgium.

5 University of Antwerp, Laboratory of Adsorption and Catalysis (LADCA), Universiteitsplein 1, 2610 Wilrijk, Belgium.

An effective approach to mitigate increasing levels of greenhouse gas emissions, involves the conversion of harmful CO2 from the atmosphere into valuable chemical building blocks through the process of photoelectrochemistry (PEC). For this application, stable semiconductor photoelectrodes with the right characteristics are required, made out of earth-abundant, non-toxic and cheap materials. Given copper’s proven activity as a reaction site for CO2 conversion, this study focusses on copper delafossites as photocathode materials.1 The delafossite structure consists of a ternary oxide characterized by the general formula ABO2, where A represents a monovalent metal cation and B a trivalent metal cation.2,3 Due to their unique crystal structure, copper delafossites can offer improved hole mobility, photostability and electrochemical stability, rendering them specifically interesting for PEC applications. In particular, CuFeO2 is a suitable photocathode, owing to its stability, conductivity and a bandgap of 1.5 eV to absorb a large portion of the solar spectrum.4 Therefore, the synthesis and materials properties of CuFeO2 were investigated. A hydrothermal route was employed to synthesize powder samples, starting from different Fe/Cu ratios. Furthermore, an aqueous solution-gel route was optimized, starting from metal citrate precursors. The CuFeO2 powders were characterized with X-ray diffraction, Raman mapping, SEM-EDX and UV-vis spectroscopy to study the phase formation, surface, composition and bandgap. The solution-gel method can also be employed to produce CuFeO2 thin films, starting from the same precursors. The thin films were also characterized by Raman and UV-vis spectroscopy. Furthermore, the produced samples were tested in a photoelectrochemical cell setup. The hydrothermal samples were tested to observe the effects of the copper and iron composition on the photocurrent, studied with LSV. These results are compared to the obtained solution-gel samples with a more porous structure. Finally, the photocurrent of the CuFeO2 thin films was also determined in the PEC setup.

REFERENCES

1.   Zhang, N., et al., Science China materials. 61(6), 771-805 (2018).

2.   Marquardt, M.A., N.A. Ashmore, and D.P. Cann, Thin solid films. 496(1), 146-156 (2006).

3.   Read, C.G., Y. Park, and K.-S. Choi, The journal of physical chemistry letters. 3(14), 1872-1876 (2012).

4.   Prévot, M.S., N. Guijarro, and K. Sivula, ChemSusChem. 8(8), 1359-1367 (2015).